Section 1.  Lesson Specification

Course Details:

| Course Title: | Weapons and Ammunition Management in UN Peace Operations In-person training |
| --- | --- |
| Course Objective | 2. T/PCCs engaged in safe and secure management of ammunition on UN peacekeeping operations. |
| Training Objective(s): | 2.1 Explain UN policies and guidelines on ammunition management. 2.2 Apply UN policies and guidelines on Ammunition Management. 2.3 Apply UN policies on secure weapon storage 2.4 Compile explosive safety cases to qualify ammunition storage licenses |

Lesson Details:

| Lesson Number and Title: | L14. Safeguarding |
| --- | --- |
| Type of Lesson / Session: | In-person, Participant Exercise |
| Duration: | 4.5 hours |

Enabling Objectives, Key Learning Points & Training Outcomes:

| Enabling Objectives: | Key learning Points: |
| --- | --- |
| 2.2.10 Compile an explosive safety case to qualify an ammunition storage license. | 2.2.10.1 Recall the contents of an explosive safety case 2.2.10.2 Recall the principles of siting an Explosive Storage Site and licensing criteria 2.2.10.3 Discuss the principles of siting an Explosive Storage Site and licensing criteria 2.2.10.4 Apply the regulations pertaining to storage conditions, storage housing, traversing and safety requirements 2.2.10.5 Apply the procedures for determining the tolerable NEQs based on the Quantity-Distance Formulae 2.2.10.6 Describe the Mapping, Yellow & Purple Lines and Directional Weapons Map 2.2.10.7 Examine how to safely control & maintain an ammunition store 2.2.10.8 Evaluate the permissible storage limits at a temporary storage area based on safeguarding good practice 2.2.10.9 Compile an explosive safety case to qualify an ammunition storage license for a temporary storage area |
| Performance Statement: | By the end of the lessons the participants will... Compile an explosive safety case to qualify an ammunition storage license. |
| Assessment Criteria: | There is no final assessment for this exercise, however, Instructors will use informal class discussion, questioning of participants and review the work by the participants to enable a positive feedback loop to be established to enhance the effectiveness of the learning. |

Resource requirements:

| Instructor to participant ratio: | 1:6 in syndicates. Maximum class size 15. |
| --- | --- |
| Interpreters: | 1 per instructor |
| Training Facilities & Equipment: | Classroom, projector and screen, flipchart, whiteboard, |
| Instructional tools & materials | - Worked out examples of ELL Matrix - Safeguarding Map - Weapons Directional Map - Imagery of explosive event(s) and impact on a camp and environs |
| Participant Resources: | - Lesson handout(s): WAMUNPOC L14 Exercise Handout - CARANA Scenario & maps |
| Training Safety Points: | Trainer is to make participants aware of course risk assessment in relation to the specific training environment. An example of Health and Safety checklist for classrooms is available here for reference here: Health and safety checklist for classrooms (hse.gov.uk) |
| Key Reference Documents: | - UN Manual on Ammunition Management - UN Weapons and Ammunition Management Policy (WAM) - International Ammunition Technical Guidelines (IATG) |

SECTION 2: LESSON PLAN

Executive summary:

During this lesson participants will compile an explosive safety case to qualify an ammunition storage license. To do this they will recall and discuss the contents of an explosive safety case and the principles of siting an Explosive Storage Site and its licensing criteria. They will then apply the regulations pertaining to storage (housing, traversing and safety requirements) and the procedures for determining the tolerable NEQs based on the Quantity-Distance Formulae. They will describe the mapping of Yellow & Purple Lines and Directional Weapons Map and then examine how to safely control & maintain an ammunition store. They will evaluate the permissible storage limits at a temporary storage area based on safeguarding good practice, before finally compiling an explosive safety case to qualify an ammunition storage license for a temporary storage area.

Setup:

Phase 1, Phase 2 (stages 1&2) and Phase 3 will be delivered to the class as a single collective.

Phase 3 (stage 3) will be conducted in small groups (6-8 participants) each with a dedicated instructor.

Conduct:

Phase 1. Introduction (Time allocation - 30 min)

- Introduce the objectives of the lesson.

- Using Case Studies and relevant imagery, discuss with the participants the importance of not only maintaining safety from an explosive event within the camp but also the impact to buildings and civilians who live within the environs of the camp.

- Ask questions of the participants to determine the likely impacts and the hazards posed to external buildings and civilians. List these impacts on the flip chart and post onto the wall.

Phase 2. Development

Stage 1 (Time allocation 50 mins) – Quantity Distances

- Introduce the concept of Quantity Distances and provide the relevant definitions to the participants. Write these on a flipchart and post it to the wall.

- Ask the participants the methods by which they can measure quantity distances when on the UN mission.

- Discuss the concept of aggregation of Net Explosive Quantity, in particular with divisions HD1.4, HD1.5 and HD1.6. Ask the participants the benefits of this aggregation and why is it useful.

- Explain the term Inside Quantity Distance and discuss how it is calculated.

- Explain the term Outside Quantity Distance, discuss how it is calculated and the distinct types of Outside Quantity Distances that need to be considered.

- Discuss the Explosive Limit License matrix, its background, the various pictograms and emphasise the need for this matrix for each Potential Explosive Site.

- Complete a worked example of an Explosive Limit License matrix for the participants, discussing each step and asking questions to the class to confirm understanding.

- By the end of this stage participants should have effectively applied the regulations pertaining to storage conditions, storage housing, traversing and safety requirements

Stage 2 (Time allocation 50 mins) – Safeguarding of Temporary Storage Areas

- Introduce the concept of Safeguarding and provide the relevant definitions to the participants. Write these on a flipchart and post it to the wall.

- Outline the requirements of safeguarding systems, the responsibilities of key personnel in completing the documentation and the preparation of the Explosive Safety Case to include safeguarding maps.

- Introduce the explosive safeguarding map to the participants and explain the significance of the yellow line and the purple line.

- Complete a worked example of preparing a safeguarding map to include the yellow line and the purple line. Discuss each step and ask questions to the class to confirm understanding.

- Emphasise the importance of maintaining and inspecting safeguarded areas to ensure that the erection of new infrastructure or movement of people/civilians into hazardous areas does not impact on the safety of the Temporary Storage Area.

- By the end of this stage participants will be able to determine the NEQ limit.

Stage 3 (Time allocation 120 mins) – Participant Exercise

- Using the CARANA scenario, provide the participants with an exercise to compile an explosive safety case to qualify an ammunition storage license for a temporary storage area (see slides for exercise detail and instructions).

- Instructors are to work closely with the syndicate groups to ensure the knowledge is affectively applied and that the participants gain a full understanding of this risk management process.

- When complete, work through the answers with the class and ask questions.

Phase 3. Consolidation (Time allocation - 20 min)

- Review Enabling objective and Key Learning Points (see Section 1), drawing out any common themes in the participant

- Look ahead to the next lesson of the course:

  - Secure weapon storage