Instructor notes:

Relocation of Unit to Existing UN Camp

| Enabling Objectives | Tasking | Learning Objectives |
| --- | --- | --- |
| Compile an explosive safety case to qualify an ammunition storage license for an existing UN compound. By the end of this training session the participant will Compile an explosive safety case to qualify an ammunition storage license for an existing UN compound | Develop an Ammunition Storage plan and supporting Explosive Storage Case for the 1 Battalion to occupy and hold Xalkaa long term | Analyse the explosive hazards on a site when taking over a T/PCC compound Analyse the condition of existing ammunition stocks Analyse the controls required to mitigate the explosive risk Analyse all safeguarding requirements for the safe storage of a UN T/PCC contingents’ ammunition Evaluate the internal and external quantity distances relevant to an existing UN compound Compile a suitable explosive risk assessment for ammunition management in temporary storage on a UN peacekeeping operation Compile an explosive safety case to qualify an ammunition storage license for a temporary storage area |

Background: Following an escalation of fighting in the Sector 3 area there has been a major increase of IDP from Rimosa towards Xalkaa. As an immediate response the Force Commander tasked Sector 3 Commander to reinforce the UN presence in Xalkaa. To do this, a Mechanised platoon was co-located at the company level UN post on the northern edge of Xalkaa. As the situation has continued to deteriorate the UN Country team have agreed that the UNHCR will increase its presence and create a Humanitarian Hub for other UN Agencies and NGOs to operate from. The Force Commander has agreed that this site will be located next to the mission base. The force will not provide access control or perimeter security for the Humanitarian Hub but they respond in the event of security issues. The SRSG has requested the Force Commander to evaluate the force posture in the area as it is expected that Xalkaa will now become a major centre of gravity for the UN and international efforts. The Force Commander has decided to assign the newly deployed Mechanised Infantry Battalion of Sector 3 with the task.

The unit will be replacing the existing light Infantry Company which has been in the position to date. The unit will also be tasked with increasing the security presence at the airstrip. Previously the light Infantry company would deploy to secure the airstrip only when a UN flight was present or when there was a UN plane on the ground overnight. In the past week the World Food Program (WFP) storage at the airstrip was looted and UN property (food aid) was stolen which is now on sale in the black market around Xalkaa. The local population are now upset as this has reduced the price of locally grown produce and there is uncertainty in the mission about how the increased UN presence will be received. The commander's intent is for the airstrip to become a new UN position of company strength. However, due to the uncertainty of the local situation there is no possibility to increase the footprint of the UN assigned space in either the main base north of Xalkaa or at the airstrip location.

- Main Base 600m by 300m

- Airstrip 400m by 200m

Additional input (add new information below approximately 1 hour into the task)

The Force has been approved to increase the space occupied at the Airstrip camp to 500m by 300m but this must also allow for the necessary support for two attack helicopters which will be based at the location. These helicopters will require three ISO containers for ammunition, one storing 500kg of HD 1.3, another container storing 400kg of HD 1.2.1 and another container storing 300kg of HD 1.1. The ESC needs to include this new information.

Instruction for DS: Deliver the narrative brief to the participants and provide the ammunition schedule. Mention IATG 08.20 – should it be used in this case?

Reaction Required:

Prepare an explosive safety case.

Annexes: (could include but are not limited to the examples below)

- Safeguarding Map

- Ammunition Breakdown by ESH

- Draft Explosive Limit License per ESH

- Engineering Works Request

  - Estimated Bill of Quantities for works required.